FAQs/Gotchas Documentation – EEG Based Error Potentials

Frequently Asked Questions

Q1: How does the ErrP Visualizer work?

A: The ErrP Visualizer is a desktop application for analyzing Error-Related Potential (ErrP) brain signals from EEG data. Here's the workflow:

- Load Data: The application has the ability to take in EEG data with the Record EEG button and the ability to select prerecorded EEG data. The Record EEG button takes you to a window where, using an OpenBCI Ganglion device, you can complete a flanker task, a standard cognitive paradigm that elicits ErrP signals by flashing arrows and asking you to type the correct one. To use prerecorded EEG data, simply drag-and-drop or browse for a .set or .csv file.

- Auto-Processing: CSV files are automatically converted to the standard EEGLAB format (.set) with proper channel locations and sampling rates.

- Configure Visualization: Choose your graph type (time series, topographic map, or joint maps), select time windows, and pick specific channels to analyze.

- Visualize: Click the Visualize button (or press Enter) to generate scientific plots showing brain activity patterns.

- Analyze: Look for ErrP signatures after error responses.

Q2: What do the different graph types show?

A:

- ErrP Time Series: Shows amplitude (microvolts) over time for selected channels. Use this to see the waveform shape and identify ERP components. The "Display Events and Responses" checkbox highlights the ERN/Ne (50-150ms, blue) and Pe (200-400ms, green) time windows with hover annotations.

- Topographic Map: Shows spatial distribution of brain activity at specific time points (e.g., 100ms, 200ms, 300ms). Warmer colors = more positive voltage; cooler colors = more negative. Requires ≥19 channels.

- Joint Maps: Combines both views, time series on top and topographic maps on bottom, at selected times below. Best for comprehensive ERP analysis showing both temporal dynamics and spatial patterns.

Q3: Why isn’t my EEG data showing any clear ErrP signals?
A: There are several common reasons why ErrP signals may not appear clearly:

- Too few trials: ErrPs are small signals that require averaging across many trials (typically 80–100+) to become visible.

- Low signal quality: Poor electrode contact or placement (especially on the forehead) can introduce noise and obscure the signal. Make sure electrodes are secure and properly positioned.

- Incorrect task performance: ErrPs are only generated when errors occur. If you made very few mistakes during the Flanker Task, there may not be enough error trials to produce a clear signal.

- Incorrect channel selection: ErrPs are strongest in frontal regions (e.g., center forehead). Make sure you're visualizing the relevant channels.

- Timing expectations: The ERN appears around 50–150 ms and the Pe around 200–400 ms after the response — check that you are looking in the correct time window.

Gotchas

Installation

- Issue: ModuleNotFoundError: No module named ‘src’

Cause: Running the app from the wrong directory or project structure changed

Fix:	cd path/to/errp-visualizer

uv run -m src.main

- Issue: uv: command not found

Cause: UV package manager not installed

Fix:	pip install uv

- Issue: PyQt5 fails to install on macOS

Cause: Missing system dependencies

Fix: 	xcode-select --install

uv sync

- Issue: Qt platform plugin could not be initialized

Cause: Missing Qt platform libraries

Fix: 	uv pip uninstall PyQt5

uv pip install PyQt5

External Resources

- Issue: UV package manager conflicts with existing pip installations

Cause: Mixing UV and pip in the same environment

Fix: use UV exclusively; don’t mix ‘pip install’ with ‘uv add’